NAMA

Inter-Urban Rail Bangladesh

| Title | NAMA Rail Bangladesh |
| --- | --- |
| Date Version number | 06/11/2016 1.1 |
| Sector, sub-sector | Transport, inter-urban |
| Country, region | Bangladesh, entire country |
| Responsible entity | Bangladesh Railway |
| NAMA formulating entity | Grütter Consulting, jgruetter@gmail.com |
| Time frame | 2018-2030 |
| Objective | Reduce significantly inter-urban transport GHG and criteria pollutant emissions in Bangladesh |
| Transformational change | Revert the trend of a declining mode share of rail in passenger and freight transport |
| Main components | Investment in expanding and upgrading rail services |
| Estimated cumulative GHG reduction 2018-2030 | 3.03 MtCO2e avoided WTW including Black Carbon |
| Sustainable Development Impact 2018-2030 | 235 avoided fatalities from transportation 735 million litres of diesel saved 850 tons PM and 19,500 tons NOx avoided 511 MUSD saved in social and environmental costs |
| Finance | Investment requirement of 30 billion USD of which 13.6 billion USD in the period 2015-2030 |

The NAMA Rail Bangladesh was developed by Grütter Consulting under contract from ADB. The Nordic Development Fund (NDF) is the financier of this assignment.

# Contents

Abbreviations	5

Summary	6

1.	Background	9

2.	Country Ownership	11

3.	Snapshot of the Rail Sector in Bangladesh	13

4.	Objectives, Targets and Actions of the NAMA	14

5.	Transformational Change	17

6.	GHG Mitigation Impact	19

6.1.	Scope	19

6.2.	Boundaries and Gases Included	19

6.3.	Cause-Impact Relationship	20

6.4.	Methodological Approach	21

6.5.	Parameters and Values Used	23

6.6.	GHG Mitigation Impact	25

6.7.	Indirect Effects	26

6.7.1.	Sources	26

6.7.2.	Road/Rail Construction Related Emissions	27

6.7.3.	Vehicle Manufacturing Emissions	27

6.7.4.        Congestion Change Impact	28

6.7.5.        Summary Indirect Emissions	29

7. Sustainable Development Impact	30

7.1. Overview	30

7.2. Economic Impacts	31

7.3. Social Impacts	31

7.4. Environmental Impacts	32

7.5. Summary SD Impacts	32

8. Finance	34

9. MRV	36

9.1. National Context	36

9.2. Monitoring	36

9.3. Institutional set-Up	38

10. NAMA Implementation Arrangements	39

Annex 1: Calculations	40

A1.1: General Input Parameters	40

A1.2: Vehicle Parameters	40

A1.3: Emission Factors Per Mode Passenger Transport	41

A1.4: Emission Factor per Mode Freight	41

A1.5: Baseline	42

A1.6: Mitigation Impact	42

A1.7: Rail and Road Construction Parameters	42

A1.8: Indirect Emissions Rail and Road Construction 2030	43

A1.9: Vehicle Manufacturing Parameters	43

A1.10: Indirect Emissions Vehicle Manufacturing 2030	43

A1.11: SD Impact on Fuel Savings	43

A1.12: SD Impact on Transportation Fatalities and Accidents	44

A1.13: SD Impact on Air Pollution	44

A1.14: Cumulative SD Impact	44

Annex 2: MRV Parameters	45

A2.1. Result Indicators	45

A2.2. Annually Monitored Parameters	47

A2.3. Parameters Monitored Every 5 Years	48

A2.4. Default and Literature Based Parameters	50

Annex 3: Stakeholder Assessment	53

References	60

# Abbreviations

AC	Air Conditioning

ADB	Asian Development Bank

BAU	Business as Usual

BC	Black Carbon

BR	Bangladesh Railway

BRT	Bus Rapid Transit

BRTA	Bangladesh Road Transport Authority

BUR	Biennial Update Reports

CDM	Clean Development Mechanism

CNG	Compressed Natural Gas

DOE	Designated Operational Entity

EF	Emission Factor

GCF	Green Climate Fund

GDP	Gross Domestic Product

GHG	Greenhouse Gases

GWP	Global Warming Potential

HDV	Heavy Duty Vehicle

IEA	International Energy Agency

INDC	Intended Nationally Determined Contribution

IPCC	Intergovernmental Panel on Climate Change

MOEF	Ministry of Environment and Forests

MRTS	Mass Rapid Transit System

MRV	Monitoring, Reporting, Verification

NAMA	Nationally Appropriate Mitigation Actions

NCV	Net Calorific Value

NDF	Nordic Development Fund

NPV	Net Present Value

OM	Operating Margin

PCU	Passenger Car Unit

PM	Particle Matter

pkm	passenger-kilometre

PT	Public Transport

QA	Quality Assurance

SCC	Social Cost of Carbon

SD	Sustainable Development

TA	Technical Assistance

tkm	Ton-kilometre

TTW	Tank-to-Wheel

UNFCCC	United Nations Framework Convention for Climate Change

WTT	Well to Tank

WTW	Well-to-Wheel

# Summary

1. GHG emissions of Bangladesh in 2011 of the power, industrial and transport sector were 64 MtCO2e and are expected to increase under Business as Usual (BAU) by 2030 to 234 MtCO2e representing 69% of total estimated emissions of 339 MtCO2e. Transport emissions are projected to increase from 5.5 MtCO2 in the year 2005 to 11 MtCO2e in 2014 to 37 MtCO2e in 2030.

2. Bangladesh’s Intended Nationally Determined Contribution of 2015 has as unconditional contribution a reduction of GHG emissions compared to BAU by 2030 of 5% in the power, transport and industry sectors. Within the INDC modal shift from road to rail is an important mitigation strategy. The National Sustainable Development Strategy of 2013 includes improvement of railway linkages as transport strategy with a potential to mitigate GHG emissions. The Perspective Plan of Bangladesh 2010-2021 of 2012 includes expansion and improvement of the railway system to increase the market share of freight and passenger transport of rail. The Railway Master Plan focuses its attention on the main corridors to regain lost market share and improve its competitive position versus road. The national climate change policies, development strategies and the relevant transport strategies all point to the importance of developing further rail transport.

3. Bangladesh Railway has as of 2016 a network of 2,880 km of routes. Road has become the dominant mode of transport with nearly 90% of passenger transport in the year 2006 with rail only accounting for 4% whilst 1976 the rail share was still 30%. In the freight sector a similar trend can be observed with 80% of tkm being transported by road and 4% by rail. Also here 1976 the rail share was much higher with 28%.

4. The overall objective of the NAMA is to increase the role of railway in the transport sector of Bangladesh. This shall be realized through expansion of railway infrastructure and increased railway efficiency. The focus is on inter-city passengers and long-haul freight along specific corridors as these are the areas where rail is considered to have a potential competitive advantage.

5. The NAMA transformational change is towards reverting the trend for decreasing rail importance in long-haul passenger and freight transport in Bangladesh thereby fostering a low-carbon transport solution.

6. The scope for the NAMA is interurban passenger and freight transport within Bangladesh. Greenhouse gases included are CO2 and Black Carbon (BC). A separation is made in direct emissions arising from fossil fuel combustion and indirect emissions from upstream fuel fossil fuel extraction, refinery and transport both of which are calculated.

7. The major cause of the GHG impact of the rail investment programme is towards mode-shift from high emitting transport means towards low emitting transport means. The mitigation impact is a resultant of differential GHG emission factors per pkm and tkm of rail versus road transport as can be seen from the following graph.

Figure 1: Emission Factor per pkm and per tkm Road versus Rail, Bangladesh 2015 (gCO2, TTW)

8. The GHG mitigation impact increases over time due to gradual implementation of the different actions planned. GHG reductions of around 3 million tCO2e can be achieved cumulative over the time period of 2017-2030 or around 450,000 tCO2e per annum as of 2030. As the railway investments have a life-span of minimum 30 years the total impact over the technical life-span is estimated at 14 million tCO2e avoided. 25% of the emission reductions are due to freight and 75% due to passenger movement. As of 2030 transport emissions could be reduced by around 1% relative to the INDC projected baseline.

9. The major indirect GHG emission source apart from upstream fuel emissions for the baseline represent the road vehicle manufacturing emissions. These represent 80% of total indirect baseline emissions. Total indirect baseline emissions for the year 2030 represent 2.5% of total baseline GHG emissions Well-to-Wheel incl. Black Carbon i.e. are not significant. The major indirect emission source apart from upstream fuel emissions for the NAMA scenario represent the rail construction emissions with 80% of total indirect emissions. Total indirect NAMA emissions for the year 2030 represent less than 8% of total NAMA GHG emissions Well-to-Wheel incl. Black Carbon. The net additional indirect GHG emissions are for the year 2030 4,000 tCO2 which represent less than 1% of projected emission reductions for the same year. This is marginal and therefore the indirect emissions are not further included with exception of the Well-to-Tank fossil fuel emissions.

10. The following table shows the projected cumulative Sustainable Development impact for the NAMA for 2017-2030 using constant fuel prices and damage costs.

Table 1: Projected Cumulative Sustainable Development Impact Period 2017-2030 (USD of 2015)

| Parameter | Impact |
| --- | --- |
| Diesel fuel savings | 753 million litres |
| Reduction of traffic accident fatalities | 235 persons |
| PM reduction | 848 tons |
| NOx reduction | 19,527 tons |
| Monetary value of fuel savings | 361 million USD of 2015 |
| Monetary value of reduced accidents | 98 million USD of 2015 |
| Monetary value of reduced air pollution | 51 million USD of 2015 |
| Monetary value total | 511 million USD of 2015 |

The cumulative SD impact in monetary terms is around 0.5 billion USD. In annual terms in 2030 the impact is 75 million USD savings. The NAMA has thus a significant impact on the well-being and the sustainable development of Bangladesh.

11. In terms of finance the rail Master Plan on which the activities and the NAMA is structured consists of 235 projects costing USD 30 billion in the period 2010 to 2030. Phase I until 2015 thereby had an investment volume of 16.41 billion USD. The NAMA has as major financial benefits the net income from its services in the passenger and the freight sector. In economic terms the Sustainable Development savings from air pollution, fuel savings and reduced accidents account for a benefit of around 3 billion USD over the investment life-span of 40 years i.e. this covers around 10% of the investment costs.  The investment cost per ton CO2 abated including economic benefits is around 1,660 USD/tCO2e. This is not the CO2 abatement cost as the rail project has income and benefits apart from GHG reductions. Finance for the NAMA could be structured basically through the Green Climate Fund with a blending of national and international resources and of credit and grant facility.

12. Within the national system for GHG inventory of Bangladesh MOEF is the focal point for reporting GHG emissions. The monitoring system for the NAMA primarily tracks GHG emissions and the sustainable development impact of inter-urban rail transport next to NAMA implementation information. Data can serve for Biennial Update Reports (BURs) as explanatory source of information concerning mitigation achievements.

# Background

GHG emissions of Bangladesh in 2011 of the power, industrial and transport sector were 64 MtCO2e and are expected to increase under Business as Usual (BAU) by 2030 to 234 MtCO2e representing 69% of total estimated emissions of 339 MtCO2e.

The 2nd National Communication reported total transport emissions for the year 2005 of 5.5 MtCO2. For 2014 transport emissions are estimated at around 11 MtCO2e based on a top-down calculation using fuel consumption data. Disaggregated fuel consumption data of Bangladesh shows for 2014-15 a gasoline consumption of 508 Mt and a diesel fuel consumption of 3,864 Mt. 100% of gasoline fuel consumption is attributed towards the transport sector as is done also in the 2nd National Communication. In diesel however the transportation sector only consumes around 54% of total diesel as is shown in the figure below, therefore resulting in 2,087 Mt of diesel used for transportation.

Figure 2: Diesel Usage by Sector Bangladesh (2013-2014)

Source: Bangladesh Petroleum Corporation; see http://www.bpc.gov.bd/contactus.php?id=30

Additionally, the transportation sector consumed 2013 around 43,000 TJ of CNG. Based on the NCV and EFCO2 of fuels as published by IPCC total transportation emissions for 2014 result in around 10.6 MtCO2. The INDC forecasts transport emissions under BAU for 2030 of 37 MtCO2e which however appear to be rather on the high end.

Figure 3: GHG Emission Trend Transport Sector Bangladesh

Source: 2005 MOEF (2012), 2014: calculation by Grütter Consulting; 2030 MOEF (2015)

Road has become the dominant mode of transport with nearly 90% of passenger transport (in terms of pkm) in the year 2006 with rail only accounting for 4% and inland waterways for 8%. 1976 the rail share was still 30% and the inland waterway share 16%. In the freight sector a similar trend can be observed with 80% of tkm being transported by road, 16% by inland waterway and 4% by rail. Also here 1976 the rail share was much higher with 28% and the inland waterway share was 37%.

Figure 4: Trends Mode Share Rail Freight and Passenger Transport (pkm and tkm)

Source: Ministry of Communications cited in GED, 2010, Table 2.8

The number of registered vehicles in Bangladesh was as of 02/2016 2.5 million units of which 1.4 million motorcycles (57% of all vehicles), 290,000 private passenger cars, 230,000 auto rickshaws, 150,000 buses (of which 87,000 microbuses, 27,000 minibuses and 37,000 large units), 119,000 trucks, and 45,000 taxis. Vehicle registration categories also include Jeeps and pick-ups as separate categories (totalling some 122,000 units) which are eventually also private cars. Total vehicle numbers have increased by factor 2.5 in the last 5 years with a large increase of motorcycles and a tripling of private passenger cars. Around 40% of vehicles are registered in Dhaka.

# Country Ownership

Bangladesh’s Intended Nationally Determined Contribution of 2015 has as unconditional contribution a reduction of GHG emissions compared to BAU by 2030 of 5% in the power, transport and industry sectors. Within the INDC following conditional mitigation options for transport are addressed:

- Modal shift from road to rail and in general to public transport (PT) in urban passenger transport with the goal of a rail mode share in urban passenger traffic of 20% by 2030;

- Improved traffic management resulting in less congestion with an expected improvement by 2030 of 15% more efficient vehicle operations. This shall be achieved inter alia through the construction of expressways and investment in PT;

The 7th 5-year plan 2016-2020 of 2015 includes as main relevant goals related to GHG mitigation in transportation to address urban congestion with Mass Rapid Transit Systems (MRTS) and to foster rail and inland waterway shipping.

The National Sustainable Development Strategy of 2013 includes as transport strategies with a potential to mitigate GHG emissions:

- Improve railway linkages and expand rail capacity.

- For Dhaka and subsequently Chittagong give priority to pedestrians and develop a MRTS based on Bus Rapid Transit (BRT) and rail-based mass transportation as well as increase the bus size on trunk routes.

The 2nd National Communication of 2012 identifies following potential mitigation measures in the transportation sector:

- Urban transport planning and traffic management to decrease congestion;

- Urban MRTS;

- Vehicle maintenance and Eco Drive;

- Expansion and modernization of railways and a mode shift from road to rail and from road to waterways.

The Perspective Plan of Bangladesh 2010-2021 of 2012 includes following strategies:

- Expand and improve the railway system with the goal of increasing the market share of freight and passenger transport of rail;

- Establish a modern MRTS in Dhaka and increase the number of modern high-capacity buses.

Bangladesh’s Climate Change Strategy and Action Plan of 2009 identifies as mitigation area in the transportation sector following actions:

- Promotion of low cost PT;

- Improved energy efficiency of vehicles;

- Substitution of fossil fuels with biofuels.

The National Integrated Multimodal Transport Policy of 2013 has recognized the need for shifting investment from the road network towards the rail sector and includes following main elements relevant for low-carbon transport:

- Improved integration of transport modes;

- Establishment of a MRTS in Dhaka including smart ticketing, integration of different PT services and improved accessibility;

- Improved land-use planning and traffic management including parking policies and regulatory measures (e.g. charging levy);

- Increased investment in railway and inland waterways.

The Railway Master Plan focuses its attention on the main corridors where 90% of traffic takes place. In these corridors and especially in certain freight commodities Bangladesh Railways (BR) wants to regain lost market share and improve its competitive position versus road. 235 projects costing 30 billion USD to be implemented during the period 2010 to 2030 have been included in the Rail Master Plan. The long-term Master Plan focuses on the infrastructure requirement and is expected to guide the overall development of BR in the foreseeable future.

The national climate change policies, development strategies and the relevant transport strategies all point to the importance of developing further rail transport. This is thereafter structured within the railway master plan which lays out the required infrastructure needs to implement the policies and strategies. Therefore, the rail NAMA is well embedded within the national policies and strategies and commensurate with the national development goals.

# Snapshot of the Rail Sector in Bangladesh

BR has as of 2016 a network of 2,880 km of routes consisting of three different gauges of which 2,656 km are under operations. 8% of the route has double track or partial double track. Since 1970 the total route length has only varied marginally. The following figures show development of passenger and freight traffic since the 70ties.

Figure 5: Freight and Passenger Movement BR

Source: BR, various Yearbooks

The freight movement has remained constant since the mid seventies at around 700 million tkm per annum transporting 2.5 to 3.5 million tons of goods over a lead distance of 250 km. Passenger movement has increased in the last decade reaching 8 billion pkm with around 65 million passengers and a lead distance of 130 km. Compared to the seventies the passenger number has dropped but the lead distance has more than doubled thus resulting in increased pkm. In the same period however the population of Bangladesh increased by the factor 2.5 from around 65 million inhabitants in 1970 to nearly 170 million in 2015 whilst GDP at constant prices increased by factor 6 in the same period.

BR manages around 300 diesel locomotives, 1,500 passenger carriages and nearly 10,000 freight wagons. Wagons carry a maximum load of 40 tons. The average freight train speed is 23 km/h. The average speed of passenger trains is around 32km/h. Low punctuality is a matter of great concern for rail customers.

Rail share of freight transport has dropped significantly in the last few decades and now reaches only around 4% in terms of passenger and freight numbers compared to more than 80% of road transport.

# Objectives, Targets and Actions of the NAMA

The overall objective of the NAMA is to increase the role of railway in the transport sector of Bangladesh. This shall be realized through expansion of railway infrastructure and increased railway efficiency. The focus is on inter-city passengers and long-haul freight along specific corridors as these are the areas where rail is considered to have a potential competitive advantage.

The strategic approach includes:

- Focus on major corridors (existing and new proposed corridors);

- Focus on major commodities;

- Gauge rationalization focusing on Broad Gauge for the entire country;

- Unlocking of capacity constraints e.g. through double tracking;

- Construction of important new links;

- Re-commissioning of tracks;

- Priority on maintenance and rehabilitation of infrastructure;

- Maintenance and procurement of rolling stock;

The main infrastructure targets and actions are summarized in the following table.

Table 2: Infrastructure Targets and Actions Planned

| Corridor | Actions |
| --- | --- |
| Corridor 1: Dhaka-Chittagong | 209km of double tracking; construction of 3rd and 4th line from Dhaka to Tongi and potentially line extension of 102km to Cox’s Bazar. |
| Corridor 2: Chilahati-Ishurdi-Khulna-Mongla | Rehabilitation and modernisation of 52km section of the corridor and extension to Mongla Port. |
| Corridor 3: Dhaka-Bangabandhu Bridge-Darsana/Benapole | Line rehabilitation and a partial construction of double line. |
| Corridor 4: Dhaka- Bangabandhu Bridge-Rajshahi-Ronapur and -Ishurdi-Parbatipur-Chilahati/Birol | Rehabilitation and replacement of major infrastructure along the line and partial double line. |
| Corridor 5: Dhaka-Sylhet/Shahbazpur | Re-commissioning of part of the line; gauge conversion; infrastructure rehabilitation |
| Corridor 6: Dhaka-Bangabandhu Bridge-Sirajganj/Rojpur-Bogra-Lalmonirhat-Burimari | 66km of new rail line and rehabilitation and modernisation of other sections |
| Corridor 7: Dhaka-Mawa-Jajira-Bhanga-Jessore-Khulna-Mongla / Jessore-Benapole / Bhanga-Barisal / Bhanga-Kashiani-Gopalgonj-Tungipara | New corridors with new lines. The new lines would reduce also the railway distances. It is expected that the new lines attract 1.11 million additional passengers. |
| Corridor 8: Dhaka-Mymensingh-Jamalpur-Tarakandi-Bangabandhu Bridge / Dhaka-Bhairab Bazar-Mymensingh | Rehabilitation of tracks and double line. |
| Corridor 9: Dhaka-Paturia-Douladia-Moukuri-Bara Durgapur- Pabna-Ishurdi | New construction |

Source: BR Master Plan, chapter 8

The following figure shows the planned network.

Figure 6: BR Projected Network by 2030

Source: BR Master Plan, Chapter 9

The Rail Master Plan projects an elasticity of passenger growth relative to GDP growth of 1.09 until 2020 and 1.0 thereafter. For freight a differentiation of growth rates based on commodities is made with some increasing in importance and others declining. Considering the most important commodities an elasticity of freight growth relative to GDP growth of 1.3 until 2020 and of 1.0 after 2030 is assumed. The GDP annual growth rate assumed for the period is 5.5%.

The rail expansion with additional lines, double tracking, rehabilitation and increased capacity utilization requires also significant investments in rolling stock as well as improved capacity utilization of existing rolling stock with improved maintenance. Cumulative until 2030 it is projected that around 300 additional locomotives, 2,000 additional passenger coaches and around 1,000 freight and flat wagons need to be procured.

The total investment required for the period 2010 to 2030 is around 30 billion USD.

BR is also taking steps to reduce emission of the railway system itself including:

- Replacement of old locomotives with new units with lower fuel consumption and GHG emissions;

- Usage of low sulfur diesel fuel. Whilst low-sulfur diesel will not have a direct GHG impact it does have an indirect impact as it allows for the usage of more efficient engines and reduces maintenance requirements. High sulfur diesel result in high maintenance efforts and if latter are not realized the fuel consumption and GHG emissions tend to increase.

- Improved locomotive maintenance facilities which again result in increased maintenance quality and thereby improved fuel efficiency of locos.

# Transformational Change

The NAMA transformational change is towards reverting the trend for decreasing rail importance in long-haul passenger and freight transport in Bangladesh thereby fostering a low-carbon transport solution. The structural change can best be seen in the two graphs showing passenger and freight transport in the historic perspective and under BAU and with NAMA actions.

Figure 7: Passenger Transport Rail BAU and with NAMA Bangladesh (pkm)

Source: Grütter Consulting

Compared to the baseline rail could more than double its activity level and thereby stop the declining mode share trend.

Figure 8: Freight Transport Rail BAU and with NAMA Bangladesh (tkm)

Source: Grütter Consulting

Also in freight a fundamental shift can be observed. In the last 4 decades, freight rail transport has been constant or declining resulting in a massive decrease of mode share. With the NAMA the rail sector shall grow significantly thus reverting the historic trend of decreasing mode share in freight transport. The intervention has therefore a large scaling-up potential.

# GHG Mitigation Impact

## Scope

The scope for this NAMA is interurban passenger and freight transport within Bangladesh (see following figure).

Figure 9: NAMA Scope

Source: Grütter Consulting

The time period calculated is until 2030 with expected impacts starting 2017 i.e. the baseline and the mitigation scenario is realized for the period 2017 to 2030.

## Boundaries and Gases Included

The NAMA and GHG boundary is Bangladesh. The activities are all over the country and therefore transport trips are all over Bangladesh.

GHG gases included are:

- Emission sources included are CO2. For inter-urban transport gaseous fuels are in general not used. Therefore, CH4 and N2O emissions are not accounted for as these represent in liquid fuels a marginal fraction. This approach is commensurate with registered CDM transport methodologies e.g. ACM0016.

- Black carbon (BC) and its impact on global warming based on a GWP100. A scientific assessment of BC emissions and impacts found that BC is second to CO2 in terms of its climate forcing. BC is on average 2,700 times more effective on a mass-equivalent basis than CO2 in causing climate impacts within 20 years, and 900 times more effective within 100 years. It increases global and regional temperatures when emitted into the atmosphere, where individual particles directly absorb energy from the sun and radiate it back as heat. BC also reduces the strong cooling effect of large, highly reflective surfaces such as glaciers and Arctic ice. BC is part of the particle matter (PM2.5) emissions originating from diesel engines.

Emission sources included are differentiated based on (see also for an initial discussion chapter 6.3.):

- Direct Emissions resulting from the combustion of fossil fuels (TTW i.e. Tank-to-Wheel).

- Indirect Emissions include upstream emissions for fossil fuel extraction, refinery and transport i.e. a well-to-wheel (WTW) approach. WTT upstream emissions of liquid fuels are a significant part of emissions. Other indirect emission sources for construction of rail tracks / roads, production of rail carriages / vehicles and congestion changes on roads are discussed under chapter 6.7. However, their magnitude is not considered relevant and therefore their impact is not included in the further calculations. BR only uses fossil fuels and no electricity for traction. In the case of future usage of electricity for traction the electricity based upstream emissions for production and transmission of electricity would be included as indirect emission source based on the actual operating emissions of the grid and the countries transmission and distribution losses.

## Cause-Impact Relationship

In terms of GHG mitigation the major impact of the rail investment programme is towards mode-shift from high emitting transport means towards low emitting transport means. Whilst new and more efficient locomotives might also reduce GHG emissions this is not taken into account as at the same time other measures such as increased comfort through AC carriages, increased speed and reduced passenger occupation rates to improve transport convenience might result in higher emissions per pkm and per tkm.

The causality chain shows the direct and indirect impacts expected to be caused by the actions of the NAMA. Based on the likelihood and expected magnitude of the impact the necessity of monitoring and further exploring impacts is determined.

Table 3: Causality Chain of Increased Rail Transport

| Effects |  | Magnitude of emission impact | Comment |
| --- | --- | --- | --- |
| Direct effects | Additional rail passenger and freight movement resulting in increased rail fuel consumption and GHG emissions | Large | Major impact, is quantified |
|  | Mode shift road to rail freight and passenger reducing road based fuel consumption and GHG emissions | Large (avoided) |  |
| Indirect effects | Rail track construction | Moderate | See section 6.7 |
|  | Rail locomotive, wagon and carriage production | Small |  |
|  | Upstream well-to-tank (WTT) fuel emissions due to increased rail activity | Large | Is quantified and included in calculations |
|  | Upstream well-to-tank (WTT) diesel and gasoline emissions of fuel without mode shift | Large (avoided) |  |
|  | Road construction without mode shift to accommodate for additional trucks and buses | Small (avoided) | See section 6.7 |
|  | Vehicle production emissions without mode shift | Small (avoided) |  |
|  | Congestion impact and induced traffic | Small |  |

Source: Grütter Consulting

## Methodological Approach

The GHG mitigation impact is the difference between a baseline scenario (Business as Usual) and a situation with NAMA options. To determine the baseline scenario two major parameters need to be defined:

- Activity level: which is the rail activity level in absence of NAMA actions i.e. how many pkm and tkm would rail have transported under BAU;

- Modes of transport used in absence of rail i.e. what mode would passengers have chosen and what mode of transport would be used by freight in absence of rail for the additional pkm and tkm and the corresponding emission factor per mode.

Baseline Emissions

Figure 10: Elements of the Baseline

Rail Activity Level

Emission Factor

The calculation procedure applied does not determine the entire inter-urban transport emissions. The baseline only reflects the emissions which would have been caused by the freight and passenger movement realized with the NAMA. They therefore only reflect the GHG emissions of the amount of freight and passengers transported by rail. Total transport related inter-urban emissions are significantly higher as the majority of transport is realized by road transport.

A differentiation between freight and passenger transport is made due to usage of different modes with different emission factors. However, calculation steps (see following figure) and formulas used are identical.

Figure 11: Steps to Determine Baseline Emissions

BEy=(ALNAMA,y-ALBAU,y)×i=1..nMSi,y×EFi,y+ALBAU,y×EFR,y                                       (1)

Where:

BEy	Baseline emissions of passenger and freight rail transport in the year y (tCO2e)

ALNAMA,y	Rail activity level with the planned actions in the year y (million pkm/tkm)

ALBAU,y	Rail BAU activity level in the year y (million pkm/tkm)

MSi,y	Distance weighted mode share of mode i in absence of rail in the year y (%)

EFi,y	Emission factor of mode i in the year y (gCO2e/pkm/tkm)

EFi,y=SFCi,y×NCV×EFCO2OCi,y                                                                                                                                  (2)

Where:

EFi,y	Emission factor of mode i in the year y (gCO2 per pkm or per tkm)

SFCi,y	Specific fuel consumption of mode i in the year y (g/km)

NCV	Net Calorific Value of fuel x (MJ/g)

EFCO2	CO2 Emission factor of fuel x (gCO2/MJ)

OCi,y	Occupation rate / Load factor of mode i (passengers or net tons)

Rail Activity Level

The BAU rail activity level is based on the level of passenger and freight movement as registered by BR since 2010. Freight movement has been more or less constant in the last 4 decades whilst passenger activity has shown ups and downs with a significant increase between 2000 and 2010 and a stabilization thereafter. It is clear from historic data that under BAU rail activity is not correlated to GDP growth which in the same observation period increased by the factor 6.

The rail activity level with NAMA activities is based on projections of the BR Master Plan which projects freight and passenger movement based on elasticities related to GDP growth (see chapter 4).

Chapter 5 shows the difference of freight and passenger rail movement under BAU and with planned activities.

Baseline Mode Share

The approach used to determine the baseline mode share differentiates freight and passenger movement. For freight this is straightforward and can be based on historical or observed mode shares in absence of rail. In the case of Bangladesh for freight transport the baseline mode in absence of rail is road with a mode share of more than 80%. For passenger movement a survey of rail users was conducted early 2016 by Grütter Consulting, in which passengers were asked what mode they would have used in absence of rail. This approach is also taken by the approved CDM methodologies for mode shift in transport being AM0031, ACM0016 and AM0101. It allows the user of rail to identify potential alternative transport modes under a hypothetic scenario i.e. a survey of rail users is conducted which asks passengers what mode of transport (if any) they would have used to perform this trip in absence of rail. 100% of rail users in Bangladesh thereby responded with the mode bus. This is therefore taken as the baseline mode of transport. However, the survey must be repeated in regular intervals as results might change over time with increased income levels and access to private vehicles. Also depending on distances, speed and comfort levels of trains, passenger might also use in the future planes in absence of rail. A similar survey realized among India Railway users showed e.g. that 9% of train passengers would have used a private car and 3% planes in absence of rail.

Emission Factor

The emission factor per mode (i.e. emissions per pkm or tkm of buses, trucks, rail) is calculated based on the approach used in approved CDM transport methodologies such as ACM0016 and by IPCC using the fuel consumption, the NCV, the CO2-EF of fuels and the average load factor. For BC the emissions are determined per kilometre using PM2.5 emission levels per vehicle category in accordance with the emission standards of Bangladesh, the BC fraction of PM2.5 and the Global Warming Potential (GWP) of BC.

## Parameters and Values Used

The following table summarises all parameters used for calculations.

Table 4: Parameters

| Parameter | Description | Value | Data Source | Plausibility |
| --- | --- | --- | --- | --- |
| SFCB | Specific fuel consumption buses | 43 l/100km | Grütter Consulting, 2016a based on a survey of inter-urban buses, Bangladesh; all diesel | Median driving speed 33km/h. Using COPERT for coach <18t, 0% gradient, 100% load factor, 33 km/h speed, conventional technology the resultant SFC is 41 l/100km |
| NCVD | Net Calorific Value of diesel | 43.0 MJ/kg | IPCC, 2006, Table 1.2 | Official publication |
| EFCO2,D | CO2 Emission Factor of diesel | 74.1 gCO2/MJ | IPCC, 2006, Table 1.4 | Official publication |
|  | Density of diesel | 0.84 kg/l | IEA, 2005, table A.3.8 | Official publication |
| OCB | Occupation rate buses | 32 passengers | Grütter Consulting, 2016a based on a survey of inter-urban buses, Bangladesh | Average occupation rate of 80% with 40 passenger average capacity |
| SFCT | Specific fuel consumption trucks | 37 l/100km | Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh | Median speed of 31 km/h. COPERT rigid trucks 14-18t conventional, 0% gradient, 100% load, speed 31km/h has a SFC of 37 l/100km |
| LFT | Load factor trucks | 8.3t | Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh | Average capacity utilization of 82% and average maximum payload capacity of 10 tons. The large majority of trucks are 2-axle. |
| WWTD | Well-to-tank mark-up factor diesel | 25% | JRC (2011) 22%, CEC (2007) 23%, US DOE GREET (2012) 25%, NRC GHGenius (2013) 29% | Average value of different sources |
| EFPM,B | PM 2.5 emission factor diesel coach Euro I | 0.362 g/km | COPERT, table 3-23, Tier 2 appoach | Based on average Euro I model |
| EFPM,T | PM 2.5 emission diesel HDV 7.5-16t Euro I | 0.201 g/km | COPERT, table 3-21, Tier 2 appoach | Based on average Euro I model |
| BCF | BC fraction of PM 2.5 Euro I HDVs | 65% | COPERT; table A.4.0.2 Tier 3 |  |
| GWPBC | GWP100 of BC | 900 | World Bank, 2014 | total effect including direct and Cryosphere effect |
| FCR | Diesel fuel consumption rail | 34,916 t | BR, 2015 for period 2013-14 |  |
|  | Engine-km passengers | 12.334 Mkm | BR, 2015 for period 2013-14 | Required for assigning diesel fuel consumption to passenger movement |
|  | Engine-km freight | 1.177 Mkm | BR, 2015 for period 2013-14 | Required for assigning diesel fuel consumption to freight movement |
| ALP | Passenger movement rail | 8,135 Mpkm | BR, 2015 for period 2013-14 | Used for calculating EFPKM rail |
| ACF | Freight movement rail | 677 Mtkm | BR, 2015 for period 2013-14 | Used for calculating EFTKM rail |
|  | GDP growth | 5.5% | BR, Master Plan | Trading economics based on ARIMA model has a growth rate of 6.3% until 2020; IFs forecasts an average annual growth rate of 6.9% to 2030 |
| ƐP | pkm elasticity | 1.09 until 2020 1.0 2021-2030 | BR, Master Plan | India has a similar elasticity of 1.0 |
| ƐF | tkm elasticity | 1.3 until 2020 1.0 2021-2030 | BR, Master Plan | India has a similar elasticity of 1.0 |
| MSF,T | Mode share truck for freight in absence of rail | 100% | Assumption | Road has > 80% of freight transport |
| MSP,B | Mode share bus for passengers in absence of rail | 100% | Grütter Consulting, 2016c | This value might change over time due to increased income levels and access to private vehicles |

A constant efficiency per tkm and pkm is assumed for all modes of transport. For the NAMA it is proposed to monitor core parameters required for the EF per pkm and tkm every 5 years. A constant level is assumed due to following factors which offset each other:

- Over time passengers will demand higher passenger comfort buses including AC and eventually more space. These two factors will increase specific fuel consumption of buses.

- Higher average driving speed and technology renewal will lead to a decreas of SFC of buses and trucks.

- Truck average size might increase resulting in higher amounts of commodities to be transported thus reducing SFC per tkm. On the other hand, the current average load factor is very high and will tend to drop with higher wages and more requirements on quick delivery thus increasing emission per tkm.

- Rail efficiency in terms of fuel consumption per engine-km could improve due to new technologies and improved maintenance. On the other hand, increased comfort levels for passengers including AC and more space as well as increased average speed could offset again these efficiency gains.

## GHG Mitigation Impact

The mitigation impact is a resultant of differential GHG emission factors per pkm and tkm of rail versus road transport as can be seen from the following graph.

Figure 12: Emission Factor per pkm and per tkm Road versus Rail, Bangladesh 2015 (gCO2, TTW)

Source: Grütter Consulting (see Table above for data sources)

For passenger transport rail is 3x more efficient in terms of GHG emissions and for freight transport factor 8 more efficient. Using a WTW approach and including Black Carbon road based passenger transport has 52 gCO2e/pkm versus 16 gCO2e/pkm for rail and road based freight transport results in 152 gCO2e/tkm versus 18 gCO2e/tkm when using rail.

The following figure shows forecasted baseline and NAMA GHG emissions TTW and WTW including BC.

Figure 13: GHG Emissions Baseline and NAMA, Inter-Urban Rail Transport Bangladesh (tCO2e)

Source: Grütter Consulting

The GHG mitigation impact increases over time due to gradual implementation of the different actions planned. The following table summarizes the GHG impacts. For details see Annex 1.

Table 5: GHG Impacts

| Parameter | Average annual | Year 2030 | Cumulative 2017-2030 |
| --- | --- | --- | --- |
| GHG Reduction TTW in tCO2 | 145,000 | 299,000 | 2,020,000 |
| GHG reduction WTW in tCO2e | 181,000 | 373,000 | 2,530,000 |
| GHG reduction WTW incl. BC in tCO2e | 216,000 | 447,000 | 3,030,000 |

GHG reductions of 2 to 3 million tCO2e can be achieved cumulative over the time period of 2017-2030 or around 300-450,000 tCO2e per annum as of 2030. As the railway investments have a life-span of minimum 30 years the total impact over the technical life-span is estimated at 9.5 to 14 million tCO2e avoided. 25% of the emission reductions are due to freight and 75% due to passenger movement. As of 2030 transport emissions could be reduced by around 1% relative to the INDC projected baseline.

## Indirect Effects

### Sources

Indirect emission sources are fossil fuel Well-to-Tank (WTT) emissions, road construction emissions, vehicle manufacturing emissions, and congestion change emissions. WTT emissions have already been included in above calculations and are therefore not further discussed.

It is important to take into account that indirect emissions occur under the BAU or the baseline scenario as well as under the NAMA scenario. With the baseline additional road space is required as more buses and trucks drive on roads whilst under the NAMA scenario additional rail tracks are built. The same is true for vehicle construction where additional bus and truck manufacturing have to be compared with additional locomotive and carriage manufacturing. The total impact is the differential between baseline and NAMA indirect emissions.

### Road/Rail Construction Related Emissions

This refers to additional road construction which would have been required to facilitate the movement of the baseline vehicles in the BAU scenario and to additional rail tracks under the NAMA scenario. In absence of the additional mode-switch related to the NAMA more conventional vehicles would operate which require space to move. The main component is thereby additional road lane construction. The following table shows core parameters required for an assessment of the magnitude and the direction of the impact.

Table 6: Parameters to Determine GHG Impact of Road/Rail Construction

| Parameter | Value | Source, Comment |
| --- | --- | --- |
| Emission Factor for road construction | 20 tCO2/km per lane per annum | 550 tCO2 per lane for a 4-lane road GEF; 809 tCO2 per lane (expressways) World Bank; 628-721 tCO2 per lane motorway EU Pave; EU average 500-800t per lane including construction, maintenance and operation; calculation based on 40 years of life-span |
| Emission Factor for rail construction | 40 tCO2/km of track per annum | Based on EU including stations, rail track, and bridges; based on different material life-time; annualized emissions. Does not include overhead electrification or tunnels. GEF has 875 tCO2/track i.e. around 30 tCO2/annum per km of track based only on steel and concrete for tracks. This does not include bridge and station components. |
| Road capacity per hour | 700 PCU/h | For calculation purposes 18h are assumed. Trucks and buses are counted as 2 PCUs (Passenger Car Units). Number based on single carriageway with 2 lanes with mixed traffic, pedestrian crossings and average speeds below 50km/h, width 10-12m with 20-25% of traffic being buses and trucks |
| New rail tracks | 390 | Based on map in BR, Rail Master Plan, p.122 new lines only |

Calculations are performed based on additional pkm and tkm of rail. Using the load factor of trucks/buses the additional baseline distance driven by buses and trucks can be calculated which then results in additional road space required to accommodate these vehicles. The avoided indirect emissions from road construction are for the year 2030 3,000 tCO2 whilst the additional indirect GHG emissions due to rail track construction are 18,000 tCO2 i.e. around 15,000 tCO2 additional indirect emissions are caused by the NAMA due to construction activities.

### Vehicle Manufacturing Emissions

This leakage refers to the construction and de-construction of vehicles in the manufacturing process i.e. upstream and downstream emission sources. In the baseline case more buses and trucks are required compared to the NAMA scenario whilst in latter more locomotives and carriages are demanded. The following table shows core parameters required for an assessment of the magnitude and the direction of the impact.

Table 7: Parameters to Determine GHG Impact of Vehicle Manufacturing

| Parameter | Value | Source, Comment |
| --- | --- | --- |
| Emission Factor for manufacturing of vehicle category i | 40 gCO2/km per annum bus / truck | All data based on production based emissions for the average lifetime distance driven of the vehicle assumed in 1 million km; database ecoinvent |
| Emission Factor for manufacturing of rail vehicles | 1.3 tCO2 /carriage per annum and 2.6 tCO2 / loco per annum | Based on EU figures with a life-span of 40 years for units. |
| Additional rail carriages and locos | 300 locos and 3,000 carriages | BR Master Plan, chapter 9 |

Calculations are performed based on additional pkm and tkm of rail. The avoided indirect emissions from road vehicle manufacturing are for the year 2030 16,000 tCO2 whilst the additional indirect GHG emissions due to rail vehicle construction are 5,000 tCO2 i.e. around 11,000 tCO2 indirect emissions are avoided by the NAMA in the area of vehicle manufacturing emissions.

### 6.7.4. Congestion Change Impact

This includes a speed effect (changing GHG emissions due to change of vehicle moving speed provoked through less congestion) and a possible rebound effect (additional trips due to reduced congestion). The sum of both effects can be of positive or negative nature due to the fact that GHG tailpipe emissions decrease up to a certain speed range and thereafter increase again. In the baseline more vehicles would circulate. This can result in two possible outcomes:

- Additional road-space is built i.e. additional lanes to keep the congestion at a constant level (i.e. in the baseline case more road space is built than in the project case).

- The congestion increases (i.e. is higher in the baseline case than in the project case). This would result in a speed effect (positive or negative depending on the speed with and without project) and a potential negative rebound effect. The incidence of rebound on inter-urban trips is however considered as marginal as the trip purpose itself is exogenously given i.e. not influenced by the road congestion conditions. This is different in city settings where rebound effects can have a major magnitude due to changing trip routes, changing trip times or making additional or longer trips (e.g. choosing a different shopping centre or visiting a friend).

To avoid double–counting of leakage with additional road construction and due to data difficulties in determining actual speed and congestion levels over an entire network of roads this potential leakage source is not addressed but considered within additional road construction i.e. leakage is determined assuming that the congestion level remains constant and therefore additional vehicles require additional road construction without assuming speed or rebound effects.

### 6.7.5. Summary Indirect Emissions

The major indirect emission source apart from upstream fuel emissions for the baseline represent the road vehicle manufacturing emissions. These represent 80% of total indirect baseline emissions. Total indirect baseline emissions for the year 2030 represent 2.5% of total baseline GHG emissions WTW incl. BC i.e. are not significant.

The major indirect emission source apart from upstream fuel emissions for the NAMA scenario represent the rail construction emissions with 80% of total indirect emissions. Total indirect NAMA emissions for the year 2030 represent less than 8% of total NAMA GHG emissions WTW incl. BC.

The net additional indirect GHG emissions are for the year 2030 4,000 tCO2 which represent less than 1% of projected emission reductions for the same year. This is marginal and therefore the indirect emissions are not further included with exception of the WTT emissions which represent around 25% of TTW emissions.

# 7. Sustainable Development Impact

## 7.1. Overview

Sustainable development (SD) benefits are separated in the areas of economic, environmental and social benefits. Rail transport is associated with a wide range of externalities (positive and negative) arising out of the railways reach out to the community. The positive ones are in the form of greater industrialization, trade and commercialization activities, higher social mobility, property price appreciation, greater potential for tourism, etc. The negative ones are the environmental degradation resulting from railway operations (air, water and noise effects), accident related damage, barrier effect (communities severed by rail), etc. A large part of the positive or negative externalities are however the same or in the same magnitude for road based transit e.g. greater industrialization or trade and commerce activities. Therefore, the following section will focus on SD impacts which are differential between the road based BAU option and the NAMA scenario.

The following SD benefits are looked at in detail:

- Economic: Energy security incl. conservation of energy sources and reduced dependency on imports; Reduced accident costs; reduced air pollution costs. The impact on the job market is not considered to be significant as on the one hand additional jobs are created within the rail sector whilst less jobs are required for road transport.

- Social: Improvement of quality of life including health improvements and enhancement of safety standards. The NAMA has no incidence i.e. the situation in the baseline and with a NAMA tend to be the same in terms of gender equality or on income and wealth distribution and therefore these aspects are not further discussed.

- Environment: air quality changes in terms of NOx and PM are included; The NAMA has no incidence i.e. the situation in the baseline case and with a NAMA tends to be the same concerning water, soil, waste and other pollutants. Rail transport will use reduced space compared to road transport. However, the impact is of minor nature in inter-urban transport.

The following graph relates the core SD impacts of the NAMA.

Figure 14: Sustainable Development Impact of Rail NAMA

Major Outcome

SD Impacts

reduced fossil fuel consumption and reduced GHG emissions

environment: better air quality

social: improved health and less accidents

economic: lower fuel imports & dependency

The following table shows the indicators for measuring the SD impact.

Table 8: SD Indicators

| Impact | Indicator | Comment |
| --- | --- | --- |
| Economic impact: fuel savings | - Tons of fossil fuel saved - Monetary value of fuel savings | Monetary value based on Bangladesh diesel market price |
| Environmental impact: air quality | - Reduced particle matter emissions - Reduced NOx emissions | Based on Tier 1 method and fuel savings |
| Social impact: - Savings based on NOx and PM reductions - Reduced accidents | - Monetary value of NOx damages - Monetary value of PM damages - Avoided fatalities due to accidents - Monetary value of avoided accidents | Damage from vehicle and other ground-level sources is extrapolated from a city-level database on pollution intake rates. This results in costs for damage of local air pollution for Bangladesh. |

## 7.2. Economic Impacts

Potential economic impacts of the NAMA include:

- Job creation impact;

- Improved energy security and reduced dependency on imports;

- Reduced congestion costs;

- Reduced health costs due to reduced air pollution (this is looked at under 7.4.);

- Reduced cost of loss of productivity due to accidents (this is looked at under 7.3.).

The impact on the job market is not considered to be significant ass on the one hand additional jobs are created within the rail sector whilst less jobs are required for road transport.

In urban transport projects savings due to reduced congestion tend to be the major economic impact. However, in inter-urban freight and passenger transport this is not expected to be a major impact not least due also to the projected baseline which relies on additional road construction.

Bangladesh needs to import diesel fuel causing a considerable strain on foreign exchange resources. The transportation sector accounts for 50% of the diesel usage in the country. Over the time period 2017-2030 Bangladesh can save over 750 million litres of diesel worth 360 million USD at 2015 USD prices by implementing the NAMA.

## 7.3. Social Impacts

The social benefits of a shift from road to rail are basically less accidents and an improved air quality resulting in a positive health impact. Latter impact is quantified in the next section. Bangladesh had 2008 nearly 3,000 fatalities due to road accidents of which more than 80% occurred outside the major cities. More than 50% of involved vehicles were buses or trucks. The following table shows the parameters used to determine the impact on fatal traffic accidents.

Table 9: Parameters for Determination of Fatal Accident Rates

| Parameter | Value | Source |
| --- | --- | --- |
| Registered trucks 2008 | 64,944 | BRTA, 2008, table 4-2 |
| Registered buses 2008 | 35,987 |  |
| Share of truck involvement in fatal accident 2008 | 24% | BRTA, 2008, table 4-2 incl. single and multiple vehicle involvement |
| Share of bus involvement in fatal accident 2008 |  |  |
| Fatalities non-urban traffic accidents 2008 | 2,210 | BRTA, 2008 |
| Annual distance driven truck/bus | 80,000km | Grütter Consulting |
| Total fatalities BR 2009/10 to 2013/14 | 148 | BR annual yearbooks |
| Total pkm + tkm 2009/10 to 2013/ | 43,720 Mtkm/pkm |  |
| Accident cost | 0.13 USD/l diesel | IMF, 2013 for Bangladesh USD of 2015 |
| Fatalities per million bus km | 0.20 | Calculation |
| Fatalities per million truck km | 0.10 | Calculation |
| Fatalities per million pkm and tkm rail | 0.0034 | Calculation |

The NAMA can reduce 235 fatal accidents over the period 2017-2030 (by 2030 35 persons) and reduce accident costs by a cumulative 98 MUSD or on average 7 MUSD per annum. This includes total accident cost and not only fatalities. For more details, see Annex 1.

## 7.4. Environmental Impacts

The environmental impact avoided is basically in terms of reduced particle matter and NOx emissions due to minor usage of trucks and buses. The physical and the monetary impact is calculated. The following table shows the parameters used to determine the impact on air pollution.

Table 10: Parameters for Determination of Fatal Accident Rates

| Parameter | Value | Source |
| --- | --- | --- |
| PM2.5 emissions diesel coach Euro I | 0.362 g/km | COPERT, table 3-23, Tier 2 approach |
| NOx emissions diesel coach Euro I | 8.1 g/km | COPERT, table 3-22, Tier 2 approach |
| NOx emissions diesel HDV Euro I | 5.31 g/km | COPERT, table 3-20, Tier 2 approach |
| PM2.5 emissions diesel HDV 7.5-16t Euro I | 0.201 g/km | COPERT, table 3-21, Tier 2 approach |
| Pollution cost NOx | 370 USD/t | IMF, 2013 for Bangladesh USD of 2015 |
| Pollution cost PM | 51,900 USD/t |  |

The NAMA can reduce 20,000 tons of NOx and 850 tons of PM2.5 over the period 2017-2030 (for 2030 2,900 tons of NOx and 130 tons of PM2.5 are avoided) and reduce air pollution costs by a cumulative 51 MUSD or on average 4 MUSD per annum. For more details, see Annex 1.

## 7.5. Summary SD Impacts

The following table shows the projected cumulative SD impact for the NAMA for 2017-2030 using constant fuel prices and damage costs.

Table 11: Projected Cumulative Sustainable Development Impact Period 2017-2030 (USD of 2015)

| Parameter | Impact |
| --- | --- |
| Diesel fuel savings | 753 million litres |
| Reduction of traffic accident fatalities | 235 persons |
| PM reduction | 848 tons |
| NOx reduction | 19,527 tons |
| Monetary value of fuel savings | 361 million USD of 2015 |
| Monetary value of reduced accidents | 98 million USD of 2015 |
| Monetary value of reduced air pollution | 51 million USD of 2015 |
| Monetary value total | 511 million USD of 2015 |

Source: Grütter Consulting

The cumulative SD impact in monetary terms is around 0.5 billion USD. In annual terms in 2030 the impact is 75 million USD savings. The NAMA has thus a significant impact on the well-being and the sustainable development of Bangladesh.

# 8. Finance

The rail Master Plan on which the activities and the NAMA is structured consists of 235 projects costing USD 30 billion in the period 2010 to 2030. Phase I until 2015 thereby had an investment volume of 16.41 billion USD. The following table lists projected investment costs and the potential funding source for Phases II to IV from 2015/16 to 2030.

Table 12: Projected Investments (billion USD)

| Phase | Finance GOB | External Finance | Total Cost |
| --- | --- | --- | --- |
| 2015-2020 | 1.16 | 2.45 | 3.61 |
| 2020-2025 |  |  | 5.64 |
| 2025-2030 |  |  | 4.34 |
| Total 2015-2030 |  |  | 13.59 |

Source: BR, Master Plan, chapter 9

The NAMA has as major financial benefits the net income from its services in the passenger and the freight sector. In economic terms the SD savings (air pollution, fuel savings and accident) account for a benefit of around 3 billion USD over the investment life-span of 40 years i.e. this covers around 10% of the investment costs.

The investment cost per ton CO2 abated is calculated based on a 40-year period with constant GHG reductions from 2030 onwards and total investments. This amounts to an investment cost of around 1,830 USD per tCO2e avoided. Including economic benefits, the net investment cost is reduced to around 1,660 USD/tCO2e. This is not the CO2 abatement cost as the rail project obviously has income and benefits apart from GHG reductions. The main reason for investing in rail is to transport more freight and passengers. The investment thus has monetary returns without being fully profitable. To determine the profitability, operational revenues and costs to determine the annualized loss or profit of the investment need to be included, resulting thereafter in positive or negative GHG abatement costs. The marginal abatement cost is very much dependent on price regulations within the railway sector, especially for passenger transport, where fare prices are based on considerations including accessibility and social pricing.

Support for early NAMA implementation is likely to come from bilateral funding sources and multilateral development banks. The Green Climate Fund (GCF) has become operational and has decided upon first credits and grants in 2015. Nevertheless, there is no mechanism through which NAMAs can almost automatically receive international grants or other funding as it is expected by many actors. In fact, it is highly unlikely that NAMAs will be financed only through grants and/or public funds. Hence, NAMA financing mechanisms need to be established to leverage and mobilise private investments. The major part of necessary climate investments will not come from the international realm but from domestic sources like taxes, levies, public budgets, etc. The GCF can however be critical in providing complementary resources to those of existing financial actors.

Germany and the United Kingdom have launched on December 6, 2012, in Doha the NAMA Facility to support and mobilize the financing of NAMA implementation as the first financing facility specifically aimed at NAMAs; mitigation actions are only starting to be financed as NAMAs. This facility, has as of May 2016 supported 14 projects with a maximum of 20 million Euro per NAMA i.e. the facility is not geared towards large infrastructure programs but towards capacity building, smaller projects and towards creating the conditions to access major finance. Therefore, it is not surprising that no rail project has entered the NAMA facility Fund.

For sources of carbon finance see e.g. http://www.climatefundsupdate.org

# 9. MRV

## 9.1. National Context

Within the national system for GHG inventory of Bangladesh MOEF is the focal point for reporting GHG emissions. In the transport sector only a top-down approach has been followed to the moment without further disaggregation except in the 2nd National Communication for vehicle categories.

The monitoring system for the NAMA primarily tracks GHG emissions and the sustainable development impact of inter-urban rail transport. For the national GHG monitoring system it can provide tracking of the impact of NAMA implementation and certain parameters can serve as input factor for a bottom-up estimation of GHG transport emissions. Bottom-up data again can serve for Biennial Update Reports (BURs) as explanatory source of information concerning mitigation achievements.

## 9.2. Monitoring

The following figure shows the steps required in monitoring to determine the NAMA impact.

Figure 15: NAMA Monitoring Steps

Step 1 Implementation status: The physical implementation of the NAMA is monitored and compared to projections. Core parameters include distance of new tracks built, distance of new double tracking rails and number of additional coaches, carriages and locos. This data is not required for emission reduction calculations but shows the progress of the NAMA and can be used for plausibility of the monitored activity levels and emission reductions.

Step 2 Activity level: The activity level in terms of pkm (only inter-urban transport i.e. if in the future an urban passenger rail system e.g. metro, is established these numbers are not included) and tkm is monitored together with the passenger numbers and freight tonnage and the average lead passenger and freight transport. This data is already collected and reported annually by BR.

Step 3 Energy consumption rail: The diesel fuel (and eventually in the future electricity consumption) of rail is monitored. The assignment to freight and to passenger transport can be based on the engine-km for freight and for passenger movement. In case of establishment of an urban rail system in the future (metro or LRT) the energy consumption of the urban system needs to be excluded. This data is already collected and reported annually by BR. Together with the tkm and pkm of rail and the emission factor of diesel this allows to calculate the specific emission factor per tkm and per pkm of rail.

Step 4 Baseline emission factors: Factors which are revised in regular intervals include the specific fuel consumption of different vehicles (basically inter-urban trucks and buses) and the load factor of these vehicle categories. Eventually also a passenger mode split survey of rail passengers is made again in the future. The periodic update of the baseline parameters allows for an adjustment of baseline emissions and improves the precision level of emission reduction calculations.

Step 5 Emission Reductions: Based on the values for the monitored parameters and the calculus method the emission reductions are determined.

Step 6 Sustainable development parameters: The impact on sustainable development indicators is calculated. Monitored parameters include the fatality rate of rail accidents, the diesel fuel price and in a periodic manner an update of the baseline accident rate of road transport.

The following table summarizes the different result indicators and parameters monitored for the NAMA. Annex 2 contains for each parameter details concerning measurement procedures, best practice and quality assurance (QA).

Table 13: Result Indicators

| ID | Indicator | Description |
| --- | --- | --- |
| 1 | tCO2 reduced | GHG impact or outcome indicator Calculated as TTW and as WTW including BC (Black Carbon) |
| 2 | gCO2/pkm & gCO2/tkm | Core performance indicator for rail GHG effectiveness; comparison with road performance indicator |
| 3 | pkm and tkm | Core performance indicator for absolute targets in transporting passengers and freights (absolute as well as growth rate) |
| 4 | tNOx and tPM reduced | SD impact or outcome indicator for air pollution |
| 5 | Fatalities reduced | SD impact or outcome indicator for accident rate |
| 6 | MUSD cost savings due to SD impacts | SD impact or outcome indicator expressed as sum of saved costs due to reduced fuel usage, air pollution and transportation fatalities |
| 7 | # of implementation actions | Output indicator; Includes km of new built tracks, km new built double or multiple tracks, number of new locos, carriages, wagons |
| 8 | USD invested in actions | Output indicator includes new investment in rail infrastructure |

Table 14: Annually Monitored Parameters

| ID | Parameter | Description |
| --- | --- | --- |
| 1 | FC | Fuel consumption rail |
| 2 | pkm / tkm | Passenger numbers and freight tons as well as average lead |
| 3 | RFP | Reference fuel price; required for determination of the monetized SD impact. |
| 4 | Fatalities | Rail fatalities required for determination of the SD impact |

All annually required parameters are already monitored and reported by BR i.e. no additional effort is required for this part.

Table 15: Parameters Monitored every 5 Years

| ID | Parameter | Description |
| --- | --- | --- |
| 1 | SFCB/T | Specific fuel consumption of inter-urban buses and trucks |
| 2 | OCB/T | Occupation rates of inter-urban buses and load factors of inter-urban trucks |
| 3 | MS | Mode share of rail passenger users concerning transport mean they would have used in absence of rail (distance weighted) |

The mode share survey not necessarily needs to be realized. The initial survey resulted in 100% of additional rail users taking the bus. Using the same value in the future is conservative as it can be assumed that with increased income private cars will be more common. If this potential additional emission reduction shall be quantified in the future a passenger survey would need to be realized which can however be combined with a regular BR passenger survey to monitor quality of services and customer satisfaction (in India the survey has been combined with a regular passenger survey).

Table 16: Default and Literature Based Parameters

| ID | Parameter | Description |
| --- | --- | --- |
| 1 | NCV | Net calorific value of diesel (source: IPCC) |
| 2 | EFCO2 | CO2 emission factor of diesel (source: IPCC) |
| 3 | BC | Black carbon emission factor for buses and trucks (source: Copert) |
| 4 | PM/NOx | Particle and NOx emissions for buses and trucks (source: Copert) |
| 5 | GWPBC | Global Warming Potential of Black Carbon (international literature) |
| 6 | UEF | Upstream mark-up factor WTT diesel (source: international literature) |
| 7 | Fatalities | Road fatalities related to buses and trucks (source: BRTA) |
| 8 | External costs | External costs of air pollution (USD/t NOx and per tPM) and of accidents ((source: international literature) |

## 9.3. Institutional set-Up

The monitoring reports are realized on an annual base by BR. Additional data collection is limited to every 5 years.  The report can be realized within the department which is in charge of realizing the BR annual report. The climate impact can thereby be integrated within this report as a short special section thus also making efforts public and having an annual report structure without additional publication efforts. The staff in charge will be trained by Grütter Consulting who will also realize the first monitoring report, and a monitoring spreadsheet which incorporates all calculation steps and formulas required.

An external verification of the monitoring reports could be realized if required by potential financing parties. The verification could be realized for example by DOEs accredited for the CDM sectoral scope 7 (transport).

# 10. NAMA Implementation Arrangements

The NAMA is managed and implemented by BR in coordination with MOEF. Due to the simple monitoring system no additional support beyond what is already planned within the ADB NAMA project is required.

# Annex 1: Calculations

## A1.1: General Input Parameters

| Parameter | Value | Unit | Source |
| --- | --- | --- | --- |
| NCV of diesel | 43 | MJ/kg | IPCC, 2006, table 1.2 |
| CO2 emission factor of diesel | 74.1 | gCO2/MJ | IPCC, 2006, table 1.4 |
| Density of diesel | 0.844 | kg/l | IEA, 2005 |
| NCV of gasoline | 44.3 | MJ/kg | IPCC, 2006, table 1.2 |
| CO2 emission factor of gasoline | 69.3 | gCO2/MJ | IPCC, 2006, table 1.4 |
| Density of gasoline | 0.741 | kg/l | IEA, 2005 |
| Well-to-tank mark-up factor diesel | 25% |  | JRC-Study study 22%, CEC 23%, US DOE GREET model 25%, NRC GHGenius model 29%; average value taken |
| Well-to-tank mark-up factor gasoline | 25% |  | JRC-Study 19%, CEC 27%, LLC 28%; average value taken |
| PM 2.5 emissions diesel coach Euro I | 0.362 | g/km | COPERT, table 3-23, Tier 2 approach |
| NOx emissions diesel coach Euro I | 8.1 | g/km | COPERT, table 3-22, Tier 2 approach |
| NOx emissions diesel HDV Euro I | 5.31 | g/km | COPERT, table 3-20, Tier 2 approach |
| PM 2.5 emissions diesel HDV 7.5-16t Euro I | 0.201 | g/km | COPERT, table 3-21, Tier 2 approach |
| BC fraction of PM 2.5 Euro I HDVs | 65% |  | COPERT; table A.4.0.2 Tier 3 |
| GWP100 of BC (total effect incl. direct and Cryosphere effect) | 900 |  | World Bank, 2014 |
| CO2 factor of electricity | 0.649 | kgCO2/kWh | Data for 2011 based on OM of 0.666, energy generation excl. hydro of 32.626 GWh and hydro of 0.872 GWh: IGES, 2015 for OM and generation (based on DOE) and IEA for hydro 2011 |

## A1.2: Vehicle Parameters

| Parameter | Value | Unit | Source |
| --- | --- | --- | --- |
| Specific fuel consumption diesel coach (inter-urban bus) | 43 | l/100km | Grütter Consulting, 2016a based on a survey of inter-urban buses, Bangladesh; all diesel |
| Occupation rate coach | 80% |  | Grütter Consulting, 2016a based on a survey of inter-urban buses, Bangladesh |
| Coach capacity total | 40 | passengers | Grütter Consulting, 2016a based on a survey of inter-urban buses, Bangladesh |
| BC emissions coach | 0.235 | g/km | calculation based on BC content and PM emissions coach Euro I |
| SFC trucks diesel | 34 | l/100km | all diesel; Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh |
| Average net load | 8.3 | tons | Based on survey with tkm/km of records; Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh |
| Capacity load trucks | 9.8 | tons | Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh |
| Average load factor trucks | 82% |  | Grütter Consulting, 2016b based on survey of long-haul trucks, Bangladesh |
| BC emissions trucks | 0.131 | g/km | calculation based on BC content and PM emissions HDV 7.5-16t, Euro I |
| Diesel fuel consumption | 34,916 | tons | BR, 2015; period 2013-14 |
| Engine-km passengers | 12,334,000 | km | BR, 2015; period 2013-14 |
| Engine-km freight | 1,177,000 | km | BR, 2015; period 2013-14 |
| Engine-km others | 1,489,000 | km | BR, 2015; period 2013-14 |
| Passenger km (million) | 8,135 | pkm | BR, 2015; period 2013-14 |
| Freight tkm (million) | 677 | tkm | BR, 2015; period 2013-14 |
| Average passenger trip distance | 125 | km | BR, 2015; period 2013-14 |
| Lead distance freight | 268 | km | BR, 2015; period 2013-14 |

## A1.3: Emission Factors Per Mode Passenger Transport

| Mode | GHG TTW | GHG WTW | BC | GHG WTW incl. BC |
| --- | --- | --- | --- | --- |
| Passenger cars | 105 | 132 | 0 | 132 |
| Coach | 36 | 45 | 7 | 52 |
| Train passenger | 12 | 16 | 0 | 16 |

## A1.4: Emission Factor per Mode Freight

| Mode | GHG TTW | GHG WTW | BC | GHG WTW incl. BC |
| --- | --- | --- | --- | --- |
| Truck | 110 | 138 | 14 | 152 |
| Train freight | 14 | 18 | 0 | 18 |

## A1.5: Baseline

## A1.6: Mitigation Impact

## A1.7: Rail and Road Construction Parameters

| Parameter | Value | Unit | Source |
| --- | --- | --- | --- |
| Road capacity per hour | 700 | PCU/h | Highways Agency, 1999; 2 lane 1 carriageway; 10-12m wide; speed ,50kmh; 20-25% HDVs |
| hours per day for road | 18 | h |  |
| Nu. PCU truck and bus | 2 |  | due to size |
| EF road construction | 20 | tCO2/lane/a | Hill, 2012; EUPave, 2011 |
| EF rail construction | 40 | tCO2/track/a | Hill, 2012, table 3.8 excludes tunnel and electric |
| Rail tracks BR 2013 | 390 | km | BR, Master Plan p. 122 with new lines Dhaka Jessore, Dhaka-Laksam, Corridor 6 |

## A1.8: Indirect Emissions Rail and Road Construction 2030

| Parameter | Value | Unit |
| --- | --- | --- |
| Additional road required | 169 | km |
| Additional road emissions | 3,388 | tCO2e/a |
| Additional rail tracks | 454 | km |
| Additional rail emissions | 18,144 | tCO2e/a |

## A1.9: Vehicle Manufacturing Parameters

| Parameter | Value | Unit | Source |
| --- | --- | --- | --- |
| EF for manufacturing bus/truck | 40 | gCO2/km | ecoinvent Transport Services data version 2.0, 2007 |
| EF for manufacturing loco | 2.6 | tCO2/a | Hill, 2012 |
| EF for manufacturing carriage | 1.3 | tCO2/a | Hill, 2012 |
| Additional locos | 300 | locos | BR, Master Plan, chapter 9 |
| Additional rail carriages | 3,000 | carriages | BR, Master Plan, chapter 9 |

## A1.10: Indirect Emissions Vehicle Manufacturing 2030

| Parameter | Value | Unit |
| --- | --- | --- |
| Additional GHG due to road vehicles | 15,580 | tCO2e/a |
| Additional GHG due to rail vehicles | 4,680 | tCO2e/a |

## A1.11: SD Impact on Fuel Savings

## A1.12: SD Impact on Transportation Fatalities and Accidents

## A1.13: SD Impact on Air Pollution

## A1.14: Cumulative SD Impact

| Parameter | Impact | Unit |
| --- | --- | --- |
| Diesel fuel savings | 753 | million litres |
| Reduction of traffic accident fatalities | 55 | Persons |
| PM reduction | 848 | Tons |
| NOx reduction | 19,527 | Tons |
| Monetary value of fuel savings | 361 | MUSD |
| Monetary value of reduced accidents | 98 | MUSD |
| Monetary value of PM & NOx reductions | 51 | MUSD |
| Monetary value total | 511 | MUSD |

# Annex 2: MRV Parameters

## A2.1. Result Indicators

Result indicators are calculated and not measured. The following tables give the definitions and the calculation methods. The parameters used to calculate the indicators are detailed in A2.2. and A2.3.

| ID | R1 |
| --- | --- |
| Indicator | tCO2e |
| Description | GHG emissions in tCO2e |
| Calculation method | The indicator is calculated as Tank-to-Wheel (TTW) and as Well-to-Wheel (WTW). GHG emissions are: GHG=iFCi×NCVi×EFCO2,i+EC×EFelec where: FCi Fuel consumption of fuel type i NCVi Net calorific value of fossil fuel type i EFCO2,i CO2 emission factor of fossil fuel type i EC Electricity consumption for traction EFelec Carbon emission factor of electricity production WTW includes also the WTT emissions of fossil fuels as well as Black Carbon (BC) emissions: WTW=iTTWi×UEFi+BC where: TTWi TTW GHG emissions of fuel i UEFi Upstream emission factor of fuel type i BCi Black Carbon emissions (determined per vehicle technology and fuel type) |
| Comment | See default and monitored parameters for details |

| ID | R2 |
| --- | --- |
| Indicator | gCO2e/pkm and gCO2e/tkm |
| Description | GHG emissions per passenger-kilometre and per ton-kilometre |
| Calculation method | Idem to R1 GHG emissions are calculated TTW and WTW using the same formulae but dividing it with the pkm and the tkm. The indicator is calculated for rail, trucks and buses. |
| Comment | Core performance indicator for rail effectiveness |

| ID | R3 |
| --- | --- |
| Indicator | Rail pkm and tkm |
| Description | Total passenger-kilometre and ton-kilometre per annum of rail |
| Calculation method | Based on passenger number and passenger lead and freight tons and freight lead as recorded by BR |
| Comment | Core performance indicator for achieving absolute service targets |

| ID | R4 |
| --- | --- |
| Indicator | PM and NOx |
| Description | PM and NOx emissions reduced |
| Measurement or calculation method | This can be based on Tier 2/3 method (Copert) by multiplying distance driven with the PM and NOx emission factor for buses and trucks. This needs to be adjusted based on the average Euro standard of buses and trucks circulating in Bangladesh. Latter can be based on vehicle registration statistics or surveys. The reduced emissions are based on the difference between pkm and tkm monitored and baseline divided by the average load factor bus and truck. This results in the additional distance driven per annum of trucks and buses in absence of the rail NAMA. |
| Comment | Major SD indicator |

| ID | R5 |
| --- | --- |
| Indicator | Fatalities |
| Description | Transportation fatalities reduced |
| Measurement or calculation method | The train fatalities are monitored. The road fatalities are calculated based on the default indicator of road fatalities per million bus-km and per million tkm. The additional distance driven of buses and trucks is calculated idem to R4 based on the difference between pkm and tkm monitored and baseline divided by the average load factor bus and truck. |
| Comment | Major SD indicator |

| ID | R6 |
| --- | --- |
| Indicator | MUSD |
| Description | Cost savings due to SD impacts |
| Measurement or calculation method | Sum of cost savings of reduced diesel fuel consumption (saved diesel fuel multiplied by reference cost diesel), accident cost savings (default value per km and additional km driven of buses and trucks) and avoided air pollution (based on tons of PM and NOx avoided multiplied with default cost factor per ton of NOx and per ton of PM) Diesel fuel savings are based on actual – baseline pkm divided by the bus occupation rate to determine the bus distance driven. This is then multiplied by the specific fuel consumption. The same is applied to trucks. The sum is the fuel saved. |
| Comment | Major SD indicator |

| ID | R7 |
| --- | --- |
| Indicator | Implemented actions |
| Description | List of mitigation actions implemented |
| Measurement or calculation method | Includes km of new built tracks, km new built double or multiple tracks, number of new locos, carriages, wagons |
| Comment | Major output indicator |

| ID | R8 |
| --- | --- |
| Indicator | MUSD |
| Description | Millions of USD invested in mitigation actions |
| Measurement or calculation method | Includes investment in new built tracks, km new built double or multiple tracks, number of new locos, carriages, wagons |
| Comment | Major output indicator |

## A2.2. Annually Monitored Parameters

| ID | 1 |
| --- | --- |
| Parameter: | FC |
| Unit: | Tons or litres |
| Description: | Fuel consumption of diesel for traction inter-urban rail (per annum) |
| Source of data: | BR |
| Monitoring method: | BR records |
| Measuring frequency: | Continuous with annual report |
| QA/QC procedures: | Comparison over time per engine-km |
| Comment: | For assignment to freight and to passenger transport the relation engine-km freight and engine-km passenger is taken. To convert diesel from litre to mass the density of diesel is 0.844 kg/l (IEA, 2005) |

| ID | 2 |
| --- | --- |
| Parameter: | pkm and tkm |
| Unit: | pkm / tkm |
| Description: | Passenger-kilometre and ton-kilometre on inter-urban rail |
| Source of data: | BR |
| Monitoring method: | Based on total passenger number and passenger lead and total tons transported and average lead |
| Measuring frequency: | Continuous measurement and annual reporting |
| QA/QC procedures: | Comparison with value of previous years to identify possible data recording and transmission errors. |
| Comment: |  |

| ID | 3 |
| --- | --- |
| Parameter: | RFP |
| Unit: | USD/l |
| Description: | Reference fuel price diesel |
| Source of data: | Reference price diesel based on New York Harbor Ultra-Low Sulfur No 2 Diesel Spot Price, as published by www.indexmundi.com plus 0.1 USD/l for distribution and 0.1 USD/l international transportation (IMF, 2013). |
| Monitoring method: | This reflects the base price of diesel. This is not identical to the pump price for diesel which might be higher or lower than the reference price depending on tax levels or subsidies. |
| Measuring frequency: | Annual |
| QA/QC procedures: |  |
| Comment: | Used to determine the SD financial impact |

| ID | 4 |
| --- | --- |
| Parameter: | Rail fatalities |
| Unit: | Passengers |
| Description: | Fatalities due to rail accidents |
| Source of data: | BR |
| Monitoring method: | BR records |
| Measuring frequency: | Continuous measurement and annual reporting |
| QA/QC procedures: |  |
| Comment: | Used to determine the SD impact on accidents |

## A2.3. Parameters Monitored Every 5 Years

| ID | 5 |
| --- | --- |
| Parameter: | SFCB/T |
| Unit: | l/100km |
| Description: | Specific fuel consumption of inter-urban buses and trucks |
| Source of data: | Potential sources of information: - Data from large inter-urban bus companies and logistics companies; - Reported values of literature with a transparent procedure how data was collected; - Sample measurements of trucks e.g. with installation of fuel-measurement equipment or with collection of fuel invoices and GPS route tracking |
| Monitoring method: | Reports, surveys or sample measurements |
| Measuring frequency: | Every 5 years (e.g. 2020, 2025, 2030) |
| QA/QC procedures: | Comparison with Copert value per vehicle category based on average load factor, speed, gradient and Euro standard |
| Comment: |  |

| ID | 6 |
| --- | --- |
| Parameter: | OCB and LFT |
| Unit: | Passengers per bus and net tons per truck |
| Description: | Average occupation rates of buses and average load factors of trucks |
| Source of data: | Potential sources of information: - Data from large inter-urban bus companies and logistics companies; - Reported values of literature with a transparent procedure how data was collected; - Surveys of trucks or commodity flow surveys; - Visual occupation studies for bus or on-road surveys |
| Monitoring method: | Reports, surveys or sample measurements For trucks the net load factor needs to be taken (total GVW minus tara weight of truck). This can be estimated based on percentage of load and applying this percentage to the load capacity of trucks. For buses the average occupation rate of buses based on surveys needs to be multiplied with the maximum carrying capacity of the bus. Measurements must be conducted outside urban areas. The sample must be randomized taking either all vehicles passing or every nth vehicle passing. Sampling must be done at least in 2 different geographical locations during minimum 2 days. The sample size must be sufficient for a 95% confidence level and a 5% maximum error bound of a point estimation using a simple random sample. |
| Measuring frequency: | Every 5 years (e.g. 2020, 2025, 2030) |
| QA/QC procedures: | Comparison with former values |
| Comment: |  |

| ID | 7 |
| --- | --- |
| Parameter: | MSi |
| Unit: | % |
| Description: | Distance weighted mode share of mode i passengers would have used in absence of rail (buses, cars, planes, induced traffic) |
| Source of data: | Specific survey realized of rail passengers; It is suggested to perform the survey together with other planned surveys of rail passengers e.g. concerning client satisfaction with services to reduce costs. |
| Monitoring method: | See below. Distance weighting is done using the following steps: - Based on 90% confidence level it is assessed if monitored distances from the survey can be considered as different or not. - For distances which have no significant difference the average of the modes with non-significant differences is taken. The mode share is weighted by distance by multiplying average mode trip number share with average distance per mode divided by average trip distance over all modes |
| Measuring frequency: | Every 5 years (e.g. 2020, 2025, 2030) |
| QA/QC procedures: | Comparison with former values |
| Comment: | It is optional to realize or not the survey. If no survey is realized it is assumed that 100% of rail users would have used buses in absence of rail based on the initial survey realized 2016. If BR wants to capture additional emission reductions resultant from passengers using plane or passenger cars instead of rail the survey must be realized. However, this makes only sense if the rail is competitive against planes and passenger cars from a perspective of speed, convenience and comfort. Otherwise it is suggested to not incur in the cost and complexity of a survey and continue with the default value. |

Figure 16: Main Features of Passenger Survey

| Feature | Details |
| --- | --- |
| Survey objective | Determine the baseline mode passengers of inter-urban rail would have used and determine the trip length. Latter is used to weight the mode used |
| Parameter | Main parameter: Mode used in absence of rail Secondary parameter: Distance travelled per mode |
| Target population | Passengers over 12 years of age using BR for inter-urban trips. Smaller children are excluded due to problems in answering the questions. |
| Sample frame | Based on average passengers transported per day on inter-urban rail trips entire India. |
| Relative error level (CV) | A global desired level of precision (CV) of 10% for the parameters of interest with a precision level of 90/10 is required. |
| Coverage | Bangladesh: national trips of IR all over Bangladesh |
| Size of Universe | Generally, in one day, BR mobilizes around 180,000 passengers per day |
| Sample size | Estimated at 3,000 completed surveys |
| Sample frequency | Every 5 years. No seasonality is expected. |
| Method of information collection | Basically two options exist: 1. Face-to-face surveys conducted e.g. a railway stations surveying passengers. Railway stations as well as passengers questioned need to be randomized. The random distribution allows that the sample mirrors the total population in any other non-observed variables such as age, gender, religion, personal preferences etc. 2. Phone based interviews nationwide asking persons which have recently used railways for inter-urban trips. This approach also allows for randomization and has potentially lower costs due to the large number of railway stations entailing high travel and collection costs of face-to-face surveys. |
| Trip distance calculation | Based on entry-exit station of the passenger. For road modes based on road maps and for plane based on ICAO distance calculator. |

Source: Grütter Consulting

## A2.4. Default and Literature Based Parameters

| ID | 8 |
| --- | --- |
| Parameter: | NCV |
| Unit: | MJ/kg |
| Description: | Net Calorific Value of diesel |
| Source of data: | IPCC Guidelines for National GHG Inventories, 2006, table 1.2 |
| Value(s) applied: | 43.0 |
| Comment: | Average value taken as reported by IPCC |

| ID | 9 |
| --- | --- |
| Parameter: | EFCO2 |
| Unit: | gCO2/MJ |
| Description: | CO2 emission factor of diesel |
| Source of data: | IPCC Guidelines for National GHG Inventories, 2006, table 1.4 |
| Value(s) applied: | 74.1 |
| Comment: | Average value taken as reported by IPCC |

| ID | 10 |
| --- | --- |
| Parameter: | BC |
| Unit: | g/km |
| Description: | Black Carbon emissions diesel bus and truck |
| Source of data: | Corinair Emission Inventory Guidebook/EMEP/EEA, Emission Inventory Guidebook 2013 updated 09/2014, various tables |
| Value(s) applied: | 0.0235 g/km for coaches 0.131 g/km for trucks |
| Comment: | Calculation based on PM emissions and the BC fraction within PM for Euro I vehicles (Tier 2/3 approach) BC fraction of PM2.5: 65% for Euro I HDVs (COPERT; table A.4.0.2 Tier 3); PM 2.5 emissions see below Value needs to be updated with vehicles being upgraded to Euro II, III and further standards. Also values need to be updated if truck size changes. |

| ID | 11 |
| --- | --- |
| Parameter: | EFPM/NOx |
| Unit: | g/km |
| Description: | Particle Matter and NOx emission factor diesel coach and HDV |
| Source of data: | Corinair Emission Inventory Guidebook/EMEP/EEA, Emission Inventory Guidebook 2013 updated 09/2014, various tables |
| Value(s) applied: | Coach bus: 0.362 for PM2.5 and 9.2 for NOx Truck: 0.201 for PM2.5 and 5.31 for NOx |
| Comment: | Tier 2 approach based on Euro I vehicle; trucks based on 7.5-16t Value needs to be updated with vehicles being upgraded to Euro II, III and further standards. Also values need to be updated if truck size changes. |

| ID | 12 |
| --- | --- |
| Parameter: | GWP BC |
| Unit: | no unit |
| Description: | Global Warming Potential of Black Carbon (total effect incl. direct and Cryosphere effect) |
| Source of data: | World Bank, Reducing Black Carbon Emissions from Diesel Vehicles: Impacts, Control Strategies, and Cost-Benefit Analysis, 2014 |
| Value(s) applied: | GWP100: 900 GWP20: 2,700 |
| Comment: | Values published range for GWP100 from 450 to 1,110 and for GWP20 from 1,580 to 3,870; average value is used |

| ID | 13 |
| --- | --- |
| Parameter: | UEF |
| Unit: | % |
| Description: | Upstream mark-up factor of diesel (Well-to-Tank) |
| Source of data: | JRC - Joint Research Centre-EUCAR-CONCAWE collaboration, Well-to-Wheels Analysis of Future Automotive Fuels and Powertrains in the European Context Version 3c, 2011; California Energy Commission, Full Fuel Cycle Assessment Well to Tank Energy Inputs, Emissions, and Water Impacts, 2007; LLC, Assessment of Direct and Indirect GHG Emissions Associated with Petroleum Fuels, 2009; Nylund et.al, Fuel and Technology Alternatives for Buses, VTT Technology 46, 2012; Natural Resources Canada: NRC, GHGenius model version 4.02, 2013; US DOE, GREET The Greenhouse Gases, Regulated Emissions, and Energy Use in Transportation Model Version GREET1 2012 rev2 |
| Value(s) applied: | 25% |
| Comment: | Average value of cited studies was taken |

| ID | 14 |
| --- | --- |
| Parameter: | Road Fatalities perk bus-km and per truck-km |
| Unit: | Persons |
| Description: | Road Fatalities perk bus-km and per truck-km |
| Source of data: | BRTA |
| Value(s) applied: | Bus: 0.20 fatalities per million km Truck: 0.10 fatalities per million km |
| Comment: | BRTA reports number of fatalities. The share of buses and trucks in accidents (involving single and multiple vehicles) is used to determine the participation of trucks and buses in fatal accidents (2008 this was 26% of buses and 24% of trucks). The number of fatalities are assigned to buses and trucks based on this share. The number of registered vehicles with an annual average distance driven of 80,000km is taken to determine the road vehicle accident rate per km for buses and for trucks. |

| ID | 15 |
| --- | --- |
| Parameter: | External costs |
| Unit: | USD |
| Description: | External costs of air pollution and traffic accidents |
| Source of data: | International literature; base case IM, Getting Prices Right, 2013 |
| Value(s) applied: | PM: 51,900 USD/t of 2015 USD NOx: 370 USD/t of 2015 USD Accident: 0.13 USD/liter diesel USD 2015 |
| Comment: | Values are specific for Bangladesh but in the calculation process often defaults were applied. The diesel savings calculated based on pkm buses and tkm trucks is applied. |

# Annex 3: Stakeholder Assessment

The workshop took place as part of the ADB TA Project “TA-8731 (BAN): South Asia Subregional Economic Cooperation Railway Connectivity: Akhaura-Laksam Double Track Project – Intl Firm – NAMA Component for Accessing Climate Mitigation Funds”.

The TA project aims at developing a NAMA on inter-urban rail in Bangladesh. The workshop had the following targets:

- Discuss and get feedback on the Rail NAMA, Bangladesh.

- Discuss role and climate change mitigation potential of the transport sector in Dhaka and in Bangladesh, with a special reference to the Bangladeshi INDC and the role of potential climate finance opportunities to provide additional finance to this NAMA.

The workshop was organized by Grütter Consulting and BR as part of the TA Project mentioned above.

NAMA Stakeholder Consultation Workshop

Workshop Agenda

Workshop date: 2016 October 4th ,  at 9.30 am

Workshop venue: Le Meridien Hotel

| Duration | Activity | Responsible |
| --- | --- | --- |
| 9.00 – 9.30 | Arrival and Registration | Grutter Consulting |
| 9.30-9.45 | Welcome Address | General Manager /Project Director, BR |
| 9.45-10.00 | Welcome Address | Markus Roesner ,ADB |
| 10.00 -10.15 | Inaugural Speech by Chief Guest | Director General, BR |
| 10.15 – 10.30 | ADB’s role in Transport and Climate Finance | N. Soetantri, ADB |
| 10.30 – 11.00 | Keynote Address: Bangladesh INDC, the Role of NAMAs and the Transport Sector | Mr. Ziaul Haque, MOEF |
| Tea to be served during the above address |  |  |
| 11.00 – 11.30 | Keynote Address: Railway Master Plan 2010-2030: Status and Prospects | Md. Saidur Rahman, Additional Chief Engineer, Bangladesh Railway. |
| 11.30 – 12.15 | NAMA Rail Bangladesh | J. Grutter, Grutter Consulting |
| 12.15 – 13.00 | Discussion of Rail NAMA | K. Yangzom, ADB |
| 13.00 – 14.00 | Lunch |  |

The morning session was opened by a welcome address from Md. Mozammel Hoque, General Manager of Bangladesh Railways (BR) and from Mr. Markus Roesner, Transport Specialist from the Asian Development Bank (ADB).

The first introductory presentation was from Nana Soetantry (ADB) and versed on the role of ADB in climate-related financing for sustainable transport programs. Following, keynote addresses from the Ministry of Environment and Forest (MOEF) and from the Bangladesh Railways were useful to set the stage of the discussion.

After that, Mr. Jurg Grutter from the Consultant team followed with a detailed presentation of the NAMA Rail Bangladesh, including the potential climate finance investment that could be requested under a NAMA framework. This session was followed by a long Q&A session from the floor to finish the morning session.

QUESTIONS & ANSWERS / COMMENTS

Keynote Address on Bangladesh INDC and role of NAMAs (Md. Ziaul Haque, MOEF)

- Do you have a transport emission standard in Bangladesh? (Joint TG/Mechanical from BR)

A: currently there is no emission standard from MOEF. However, there are considering introducing national standards similar to EURO standards on fuels, but not on emissions but on fuel quality.

Keynote Address on Bangladesh Railway (Md. Saidur Rahman, BR)

- Do you have a plan to improve intermodal connectivity (bus-railway)?

A: currently it is envisaged but there is no formal plan

- Do you have a plan to improve interaction with MRT and electric traction into BR?

A: they are good suggestions and they will be introduced in the future but currently there is still no formal plan and the capital investment is too high to tackle electrification of the railways.

Presentation on NAMA Rail Bangladesh (Mr. Jurg Grutter, Grutter Consulting)

- Q: how much available NAMA finance there is for transport and how much the sector contributes to national NDC?

A (MOEF): the NAMA designs really can estimate their contribution to the transport target from the NDC. However, the transport contribution to the national NDC was done on a top-down fashion and therefore is a very crude/raw estimation of how much it will contribute to the global NDC target.

- Does the NAMA Rail calculations take into account maintenance and additional costs and emissions along the whole lifespan of the railroad?

A (Grutter): yes for tracks or roads, the lifespan is always including maintenance and CO2/GHG calculations keep track of that.

- Comment on quality and fuel used for rail, as well as the need to use sustainable rail but also road.

A (Mr. Grutter): The use of diesel PPMs in relation to sulphur is an air quality problem but not a GHG emission problem. However, lower quality diesel will have higher maintenance requirements, and therefore will impact in the long term and indirect in terms of higher maintenance (which if they are not met, will conduct to lower efficiency of engines and higher GHG emissions), as well as they hinder the introduction of more modern engines/technologies.

A (Md. Haque, MOEF): Added that the impact on health benefits/reduced air pollution can also be used as levers to attract climate and sustainable development finance.

A (Ms. Yangzom, ADB): rails and road are both important for development

- Q: how we have calculated the SD benefits and how black carbon and GHG emission have been calculated?

A (Grutter): The SD was broken down on NOX and PM emissions on truck and buses and compared to diesel use of rail through the use of COPERT model. The black carbon and GHG emissions also used COPERT and results were not modelled after SNC but actually SNC is considered not very solid as it was done top-down (mostly Tier 2).

- Q: road vs. rail users: have they been compared and taken into account?

A (Grutter):No, they haven’t been consider however in economic and financial IRR and our financial calculations we incorporate the “costs” including total economic value costs so we in some way take into account the opportunity costs of investing on roads vs. rail. However is a good point and should be taken into a closer look in future steps.

- Q: climate finance will be very important and some estimates talk about 2 billion USD/year for Bangladesh. However, the estimation of the NAMA rail CF needs is only up to 350-450 mUSD. On the other side, the requirements for a NAMA in terms of MRV capabilities etc. we have to be aware that Bangladesh has really limited capacity, so this will have to be taken into account.

A (Ms. Yangzom, ADB): capacity building on MRV GHG and SD is very important. As Nana’s presentation showed, ADB is also a knowledge bank and the Railway TA has a capacity building component which can also contribute.

A (MOEF): currently MOEF is trying to getting GCF accreditation and for that will need to develop capacities on MRV. National agencies need to also be involved and training opportunities will be on the table for national agencies through enhanced GCF and UNFCCC support.

- Q: Road shipping has an “economic speed” that optimizes costs and emissions. Is this something that exists for rail?

A (BR): Maximum speed is depending on the network however the best speed is not usually related to GHG emissions.

A (Grutter): GHG cannot override economic conditions, so the important thing is that you demonstrate that at the best economic speed.

- Q: Are there adaptation measures in the railway sector?

A (Grutter): NAMA focus is on mitigation, adaptation measures are definitely important but they are not the focus of NAMAs. NAPAs (National Adaptation Plan of Actions) are the vehicle for climate proofing.

A (Ms. Yangzom, ADB): ADB is also financing adaptation and in Bangladesh indeed adaptation

- Q: Engines need to be upgraded from Tier 0 to Tier 2 or Tier 3 engines. This is not reflected in the NAMA design. Also, the higher sulphur and PM is not taken into account in relation to mid-term impact on GHG emission increase.

A (MOEF): Tier 0 is the current baseline and BAU is Tier 1, however upgrade to Tier 2 and 3 is not currently envisaged under BAU.

A (Ms. Yangzom, ADB): ADB is also financing adaptation in transport (i.e. climate-proofing transport infrastructure) and in Bangladesh indeed adaptation has to be one of the great priorities.

# References

BRTA (2008), RTA Annual Report 2008

California Energy Commission (CEC) (2007), Full Fuel Cycle Assessment Well to Tank Energy Inputs, Emissions, and Water Impacts

EMEP/EEA (2014), Corinair Emission Inventory Guidebook (COPERT Model)

EUPave (2011), Life Cycle Assessment for Road Construction and Use

Manual for Calculating Greenhouse Gas Benefits of Global Environment Facility Transportation Projects

Grütter Consulting (2016a), Inter-urban bus survey Bangladesh

Grütter Consulting (2016b), Inter-urban truck survey Bangladesh

Grütter Consulting (2016c), Rail passenger survey Bangladesh

Grütter Consulting (2014), NAMA Inter-Urban Rail, India

Highways Agency / Scottish Office Development Department (1999), Design Manual for Roads and Bridges

Hill, N. et al (2102), EU Transport GHG: Routes to 2050 II, Final Report Appendix 2: The role of GHG emissions from infrastructure construction, vehicle manufacturing, and ELVs in overall transport sector emissions

IEA (2015), Energy Statistics Manual

IMF (2014), Getting Energy Prices Right

IMF (2013), Energy Subsidy Reform: Lessons and Implications

IPCC (2006), Guidelines for National GHG Inventories

Jahan S., Sustainable and Inclusive Transport Development in Bangladesh

Joint Research Centre (JRC) (2011), EUCAR-CONCAWE collaboration, Well-to-Wheels Analysis of Future Automotive Fuels and Powertrains in the European Context Version 3c

LLC (2009), Assessment of Direct and Indirect GHG Emissions Associated with Petroleum Fuels

Ministry of Communication (2013), National Integrated Multimodal Transport Policy

MOEF (2015), Intended Nationally Determined Contributions

MOEF (2013), National Sustainable Development Strategy

MOEF (2012), 2nd National Communication of Bangladesh

MOEF (2009), Bangladesh’s Climate Change Strategy and Action Plan

Natural Resources Canada: NRC (2013), GHGenius model version 4.02

US DOE (2012), GREET: The Greenhouse Gases, Regulated Emissions, and Energy Use in Transportation Model Version GREET1 rev2

World Bank (2014), Reducing Black Carbon Emissions from Diesel Vehicles: Impacts, Control Strategies, and Cost-Benefit Analysis

World Bank (2011), Transport – Greenhouse Gas Emissions Mitigation in Road Construction and Rehabilitation

16.4 billion USD period 2010 to 2015

MOEF, 2012

BPC data

IEA, 2013

The INDC assumes for 2013 17 MtCO2e of transport emissions which are significantly higher than the fuel consumption value reported for the same year (MOEF, 2015).

Ministry of Communications cited in GED, 2010, Table 2.8

includes the category tankers

http://www.brta.gov.bd/images/files/statistics/statistics-bangladesh-feb-16.pdf

MOEF, 2015

GED, 2015

MOEF, 2013

MOEF, 2012

GED, 2012

MOEF, 2009

Ministry of Communication, 2013

http://railway.portal.gov.bd/site/page/8e5a704d-72e2-4d69-b443-21988229cbbc/Railway-Master-Plan

See BR Master Plan, chapter 2

1970 it was 2,858km

BR Master Plan Table 5-1

http://www.kushnirs.org/macroeconomics_/en/bangladesh__gdp.html#main

BR Master Plan, chapter 4; In terms of tkm a decrease from 30% to 7% over the last 30 years can be observed. A similar decreasing trend can be observed in pkm whereas of 1970 rail still had an estimated mode share of 30% which has however dropped to around 10% with road being the dominant mode of transit in freight as well as passenger movement (Ahmed, 2010, Table 3 based on Planning Commission)

BR Master Plan, chapter 7

Corridors 1 to 9

BR Master Plan, chapter 5

see Bond, 2013 or World Bank, 2014

See for quantitative estimations chapters 6.6. and 6.7.

Includes tracks, overhead lines, stations, bridges, tunnels

Includes construction, maintenance and operation

Buses and trucks

Jahan based on World Bank

http://cdm.unfccc.int/methodologies/PAmethodologies/approved

Survey realized by Grütter Consulting in India, 2013 for rail NAMA: see Grütter Consulting, 2014

For rail based on registered pkm and tkm and the total fuel consumed

Grütter Consulting, 2016a

For Euro I the value would be 37 l/100km; EMEP/EEA, 2014

Grütter Consulting, 2016b

For Euro I the value would be 31 l/100km; EMEP/EEA, 2014

EMEP/EEA, 2014

EMEP/EEA, 2014

EMEP/EEA, 2014

http://www.tradingeconomics.com/bangladesh/gdp-growth/forecast

http://www.ifs.du.edu/ifs/frm_CountryProfile.aspx?Country=BD

Grütter Consulting, 2014; calculation for period 2000 to 2011

Grütter Consulting, 2014; calculation for period 2000 to 2011

BR Master Plan, chapter 4

Range always from TTW to WTW incl. BC

Based on constant GHG reductions from 2030 onwards for 25 additional years on average (actions are implemented gradually 2017 to 2030)

Includes only construction and not road maintenance

Construction only; based on Changer model of the International Road Federation IRF

Asphalt roads at the lower end and concrete roads at the higher end for construction and the reverse for maintenance; the total of both is very similar for asphalt or concrete roads; includes construction and maintenance

Hill, 2012; EUPave, 2011; GEF, 2015; World Bank, 2011

Hill, N. et al, 2012, table 3.8

GEF, 2015

Highways Agency, 1999

ecoinvent Transport Services data version 2.0, 2007

Hill, 2012; 104 tCO2 emissions per electric rail vehicle including end of life disposal/recycling; idem assumed for diesel locos. 50% of value assumed for carriages.

Negative effect means that less traffic would occur i.e. less induced traffic

0.87 USD/l as of end of February 2016; http://www.globalpetrolprices.com/Bangladesh/diesel_prices/

USD of 2015; IMF, 2014 (USD 2015 using http://stats.areppim.com/calc/calc_usdlrxdeflator.php)

Bangladesh Petroleum Corporation; see http://www.bpc.gov.bd/contactus.php?id=30

Reference price diesel based on New York Harbor Ultra-Low Sulfur No 2 Diesel Spot Price, US$ per gallon; price as of 29.2.2016 1.07 USD/gallon (source: indexmundi) plus 0.1 USD/l for distribution and 0.1 USD/l for international Transportation based on IMF, 2013

Damage from vehicle and other ground-level sources is extrapolated from a city-level database on pollution intake rates. This results in costs for damage of local air pollution for Bangladesh

Exchange rate of 78 BDT to USD

Excludes benefits of GHG reduction; calculation based on constant benefits from 2030 onwards for 30 years and total benefits period 2015 to 2030 (gradual implementation of investments)

See for more information http://www.nama-facility.org/projects/portfolio.html

The interval of 5 years is thereby based upon slow and only minor annual changes of the parameters thus not making it relevant to measure them annually.

Exception holiday season